# Comprehensive User’s Guide

MIMO Nonlinear Model Predictive Control (MPC) Test Harness

## Executive Summary

This Python program is a self-contained, educational test harness for Multi-Input Multi-Output (MIMO) nonlinear Model Predictive Control (MPC). It allows you to run reproducible closed-loop simulations using three pre-built dynamic processes—a cross-coupled First-Order-Plus-Dead-Time (FOPDT) process, an integrating tank, and a nonlinear exothermic chemical reactor (CSTR).

The controller uses the plant model to predict future behaviour, then solves a constrained optimisation problem at each time step to find the best control moves. The optimiser (SciPy’s SLSQP) directly handles nonlinear predictions, input and output constraints, and optional soft constraints via slack variables. The program automatically detects and handles solver failures by falling back to a safe, deterministically feasible input.

Key outputs are console performance summaries and multi-panel plots showing output tracking and control signals. A set of deterministic self-tests validates the core algorithms.

This is not production-ready control software: it lacks state observers, robustness guarantees, formal stability proofs, and plant-hardware communication. It is designed for learning, algorithm prototyping, and testing of MPC concepts with a strong emphasis on clarity and correctness.

## 1. What is a MIMO Nonlinear MPC, and Why Do We Test It?

### For the Curious, Non-Specialist Reader

Model Predictive Control (MPC) is a control strategy used in everything from chemical plants to autonomous vehicles. Instead of a simple “if temperature is too high, turn down the heater” rule, MPC looks ahead: it uses a mathematical model of the process to predict how the system will behave over a future time window (the prediction horizon) and then chooses the sequence of control actions that will bring the outputs closest to their desired setpoints while respecting limits on what the actuators can do and what the process can withstand.

MIMO stands for Multi-Input Multi-Output. In a SISO (Single-Input Single-Output) system, you control one output with one input—like a thermostat controlling room temperature with a heater. MIMO means several inputs affect several outputs simultaneously, and there are interactions: increasing one input may disturb another output. The FOPDT example controls two outputs using two inputs, and changing Input 1 moves both Output 1 and Output 2.

Nonlinear means the relationships are not simple straight lines. Many real processes are nonlinear: doubling the feed flow does not double the reactor temperature, and reaction rates depend exponentially on temperature. The CSTR (Continuous Stirred-Tank Reactor) in this program is a classic nonlinear system where the model equations involve exponential terms, making prediction more challenging.

Why test it?

- Correctness: The MPC algorithm must compute control actions that respect constraints (limits on inputs, how fast they can change, and on outputs). Tests verify that the optimiser finds feasible solutions, that the predicted outputs match the model, and that fallbacks work when the optimiser fails.

- Performance: Does the controller drive the outputs to the setpoints quickly and smoothly, even when there is model mismatch (the real plant differs from the model) or measurement noise?

- Robustness: What happens if the solver runs out of iterations? The harness tests a deterministic hold-last-input fallback.

- Education: By running the demos, you can see how tuning parameters (weights, horizons, constraints) affect control quality and solver behaviour.

## 2. How to Use the Program and Interpret the Results

### 2.1 Prerequisites

- Python 3.8 or later

- Install required libraries:

bash

pip install numpy scipy matplotlib

### 2.2 Running the Demonstrations

The program is invoked from the command line. The main script is assumed to be the file test_mpc_mimo.py.

Run all standard demonstrations (FOPDT, tank, CSTR with perfect model):

bash

python test_mpc_mimo.py

This will open three plot windows (one per scenario) and print a performance summary to the console.

Run without plot windows (useful for quick text output):

bash

python test_mpc_mimo.py --no-plots

Run specific demos:

bash

python test_mpc_mimo.py --demo cstr          # nonlinear CSTR, perfect model

python test_mpc_mimo.py --demo cstr-mismatch # CSTR with plant/model mismatch, disturbance and noise

python test_mpc_mimo.py --demo mimo-fopdt    # 2×2 linear FOPDT

python test_mpc_mimo.py --demo tank          # integrating tank

Run the deterministic validation checks only:

bash

python test_mpc_mimo.py --self-test

No plots are shown; success means all internal algorithms (fractional dead-time integration, equilibrium, constraints, fallback) pass.

### 2.3 Interpreting the Console Output

After each scenario, a summary is printed, for example:

text

2x2 cross-coupled FOPDT process

-------------------------------

Output 1: final=1.00000, target=1, final error=+4.2e-06, RMSE=0.0152, IAE=0.652

Output 2: final=0.50000, target=0.5, final error=+3.1e-07, RMSE=0.0093, IAE=0.425

Solver: 65 calls, 0 failures, 0 fallbacks, mean=11.34 ms, max=25.1 ms

Maximum violations: input=0, move=0, true output=0; maximum predicted-output slack=0

- final value / target / error – how close the last measurement is to the setpoint.

- RMSE (Root Mean Square Error) – a single number capturing overall deviation; smaller is better.

- IAE (Integral Absolute Error) – total accumulated absolute error, scaled by sampling time.

- Solver statistics – number of control steps, how many failed or needed fallback, and timing.

- Constraint violations – maximum violation of hard input, move, and output limits. Ideally zero.

- slack – when soft constraints are used, the amount of temporary constraint relaxation; appears only when the process cannot respect a predicted output limit.

### 2.4 Understanding the Plots

Each scenario opens a figure with one subplot per output (top) and one per input (bottom).

Output subplot:

- Solid line: measured plant output.

- Dashed line: setpoint (reference).

- Red dotted horizontal lines: output constraints (if any).

Input subplot:

- Solid stair-step: commanded input (controller’s decision).

- If a disturbance is added to the plant input (e.g., CSTR with mismatch), an additional lighter stair-step shows the actual plant input.

- Red dotted horizontal lines: input limits.

What to look for:

- Outputs should smoothly approach the setpoint, ideally without overshoot beyond the output constraints.

- Inputs should stay within the red dotted bounds and change gradually, respecting rate limits.

- If a solver fallback occurs (rare in normal demos), the input may stall until the next successful solve.

### 2.5 The Self-Tests

These execute several unit-level checks (fractional delay, steady-state initialisation, CSTR equilibrium, full closed-loop tracking with zero fallbacks, soft constraint slack, forced fallback). A successful run prints:

text

Running deterministic self-tests...

All deterministic self-tests passed.

Failures immediately stop and print a descriptive error.

## 3. Simplified Description of the Program (For Non-Experts)

Imagine the program as a miniature chemical plant inside your computer. The plant obeys certain physics (a model). A controller tries to steer the plant to follow a recipe (setpoints) by adjusting knobs (inputs). The controller has a copy of the plant model and thinks ahead: “If I turn this knob now and then later, where will the plant be in the future? Can I stay within safety limits?” Then it computes the best sequence of knob adjustments, applies only the first one, and repeats at the next measurement.

Key pieces:

- Process models – Three virtual plants are provided:

  - FOPDT: a linear system where each input affects each output with a gain, a time constant, and a dead time.

  - Integrating tank: a simple level that accumulates the net flow; no self-regulating.

  - CSTR: a nonlinear reactor where concentration and temperature are outputs, controlled by flow and coolant temperature.

- MPC controller – The heart of the program. It receives a measurement, updates an internal bias correction, forms a mathematical optimisation problem (minimise error while respecting limits), and calls a numerical solver.

- Simulation loop – The program connects the plant and controller step-by-step. At each step, the controller reads the (possibly noisy) measurement, computes a move, the plant applies it (with possible disturbance), and the output evolves. All values are stored for later analysis.

- Metrics and plots – At the end, the program calculates performance numbers and draws graphs so you can visually check if the controller worked.

Even without deep maths, you can change the setpoints or constraints in the scenario builder and re-run to see the effect.

## 4. Deep Dive: Details for an Expert

This section assumes familiarity with MPC concepts and constrained optimisation.

### 4.1 Process Model Interfaces

All models inherit from an abstract Process class with:

- reset(y0, u0) – set internal states to be consistent with an initial output and input (steady-state offset is computed for FOPDT).

- step(u) – advance the model one sampling interval and return the new output.

- clone() – deep-copy, essential for the internal predictor to avoid corrupting the controller’s state.

#### 4.1.1 MIMO FOPDT

Implements a matrix of independent first-order paths:
τᵢⱼ * dxᵢⱼ/dt + xᵢⱼ = Kᵢⱼ * uⱼ
The output is the sum of path states plus a constant operating-point offset to allow exact initialisation at any (y0, u0). Dead time is handled exactly under zero-order hold by splitting the sample interval into the fractional portion (where an older held input is still active) and the remainder. The _history_length buffer stores enough past inputs to reconstruct delayed values. This discretisation is exact, not an approximation, for piecewise constant inputs.

#### 4.1.2 Integrating Tank

SISO integrator: dy/dt = K * u(t – θ). Fractional delay is integrated similarly: the integral of the delayed input over the sample interval is a weighted combination of the appropriate two past input values. No self-regulating term; the level can drift without bounds, so MPC stabilisation is essential.

#### 4.1.3 Nonlinear CSTR

A classic exothermic reactor with two states: concentration and temperature. Parameters are encapsulated in a CSTRParameters dataclass. The differential equations:

text

dC/dt = (F/V)(C_in – C) – k(T) * C

dT/dt = (F/V)(T_in – T) + (–ΔH/ρCₚ) * k(T) * C – (UA/(VρCₚ)) (T – T_c)

where k(T) = A * exp(–E/(R T)). Integration uses a fixed-step classic Runge-Kutta 4 (RK4) with integration_substeps per sampling interval. This ensures numerical stability and accuracy even with moderately stiff kinetics.

The equilibrium_for_temperature_and_flow method analytically solves for the steady-state concentration and coolant temperature for a given temperature and flow, used to generate a physically valid initial condition for the scenario.

### 4.2 MPC Controller Implementation

Configuration (MPCConfig):
All MPC parameters are collected in a dataclass, including:

- Horizons: prediction_horizon Np, control_horizon Nc (Nc ≤ Np).

- Weights: output_weights, move_weights, terminal_weights.

- Scaling: output_scale, input_scale (used to normalise errors and moves).

- Constraints: element-wise input_min/max, move_min/max, output_min/max.

- Soft constraints: soft_output_weights (positive weights for each constrained output) and maximum_output_slack.

- Solver settings: solver_max_iterations, solver_tolerance, constraint_tolerance.

- bias_filter: exponential smoothing factor for additive output bias.

Initialisation (reset):
Validates that u0 respects input bounds, stores the initial input as _previous_u, resets bias to zero, and saves the internal model state.

Control step (control):

- Measurement and bias update:
raw_bias = y_measured – internal_model.y
bias ← α * raw_bias + (1–α) * bias
This simple additive output-bias correction handles mild plant/model mismatch but is not an observer – it does not guarantee offset-free control for all nonlinear plants.

- Reference horizon: If a preview of future setpoints is provided (array of shape (steps, ny)), the controller uses the first Np points; otherwise a constant setpoint is held.

- Warm start: The previous plan (shifted by one step) is used as the initial guess, projected onto the feasible set of input and move constraints (sequential element-wise clipping). If no plan exists, the previous input is repeated.

- Prediction: Uses an internal cloned copy of the process model. The _predict method expands the control plan to Np by holding the last move constant, then simulates forward. The predicted output is the sum of the model output and the current bias.

- Decision vector: The optimiser works on a vector [input_plan (Nc * nu), slack (n_slack)] where slacks are only present if soft constraints are enabled. Slack variables are one per constrained output (shared across the horizon), softening the output bounds uniformly.

- Cost function:
J = Σ[ ( (ŷ – r) / scale_y )² · w_y ] + terminal_cost + Σ[ (Δu / scale_u)² · w_Δu ] + slack_cost
The slack penalty is Σ( (s / scale_y)² · w_soft ). If any prediction step or cost evaluation fails (e.g., CSTR blow-up), the objective returns 1e30, effectively rejecting that decision.

- Constraints:

  - Input bounds: enforced via SLSQP variable bounds.

  - Move rate constraints: ineq constraint function that requires Δu - move_min ≥ 0 and move_max - Δu ≥ 0.

  - Predicted output constraints: ineq function that, for each constrained output, enforces ŷ - y_min + s ≥ 0 and y_max - ŷ + s ≥ 0. The slack s (if active) is added to both, effectively relaxing the hard bound.

- Solver: SciPy’s SLSQP (Sequential Least Squares Programming) is used. It supports equality/inequality constraints and bounds. SLSQP is a gradient-based local optimiser, so convergence to a global optimum is not guaranteed, but warm starting and the simple cost structure make local minima rare in practice.

- Post-solve validation: Even if SLSQP reports success, the candidate decision is checked for finite values, correct shape, and residual constraint margin ≥ –constraint_tolerance. The plan is re-projected onto move constraints (because SLSQP may return a point slightly outside due to tolerance). If the projected point also satisfies all constraints, the first input is applied.

- Deterministic fallback: If any validation fails, the controller uses the previous input (which is guaranteed feasible for input and move constraints). This ensures a safe, predictable action even under severe numerical difficulties. The diagnostic message includes details of the failure.

- Return: A MPCStepResult object contains the command, success flag, solver diagnostics, planned inputs, predicted outputs, bias, and slack.

### 4.3 Simulation and Metrics

run_simulation executes the closed-loop experiment deterministically (same random seed reproduces noise/disturbance sequences). For each time step:

- Measurement = true output + Gaussian noise (optional).

- Controller computes move; a user-specified plant_input_disturbance is added to the command before sending to the plant.

- Plant advances one step.

- All signals are recorded.

Performance metrics are computed after the simulation:

- rmse, iae per output.

- Maximum constraint violations (input, move, true output) using _maximum_bound_violation.

- Solver failures and fallback counts.

- Timing statistics.

Plots are generated via Matplotlib, with each output and input on a separate subplot for clarity. The plt.step draw style highlights that inputs are held constant over each sample interval (zero-order hold).

### 4.4 Self-Tests

The run_self_tests function exercises:

- Exact fractional dead-time integration for FOPDT and integrator.

- Operating-point initialisation: resetting at an arbitrary non-zero state and confirming that applying the same input yields the same output (steady state).

- CSTR equilibrium consistency.

- Three standard closed-loop scenarios with tight tracking tolerance and zero constraint violations.

- Soft constraints: a trivial process where the output is permanently above its limit requires a positive slack for feasibility.

- Forced fallback: iteration limit set to 1 causes SLSQP to fail; the controller must detect this and return the previous input.

These tests ensure the core routines behave exactly as mathematically derived.

### 4.5 Design Decisions and Limitations

- No state estimator: The controller uses the most recent plant measurement directly, not an observer. Model mismatch is only corrected through the additive output bias. This simple approach works for many stable processes but cannot be proven offset-free. For unstable systems (like the integrating tank), correct initialisation and bias filter tuning are crucial.

- Gradient-based solver: SLSQP requires a smooth cost and constraints. The use of soft constraints avoids non-smooth hard constraint formulations that can cause convergence problems.

- Single internal model copy: The predictor clones the model at each control step, which is clean but adds overhead. For a real-time implementation, a dedicated predictor with roll-back capability would be preferred.

- Output-constraint softening: A single slack per constrained output is shared across the horizon. More sophisticated methods could use horizon-varying slacks, but this simplified approach keeps the decision vector small and the problem well-conditioned.

- Scalability: The optimisation grows with Nc * nu. For moderate dimensions the harness is fast enough for demonstration; real industrial problems would need custom QP solvers for linear MPC or nonlinear programming tailored to the specific process.

- No terminal constraints or cost shaping for stability guarantees. The terminal weight is a simple quadratic penalty, not a terminal set or a control Lyapunov function. As such, closed-loop stability is not formally proven, though empirical tuning yields good behaviour.

## 5. Additional Relevant Information

### 5.1 Extending the Harness

The program is designed to be extended:

- Adding a new process: Subclass Process, implement reset, step, clone. The controller will treat any Process object as a black-box simulator.

- Custom scenarios: Copy a builder function, modify setpoints, constraints, weights, or add model mismatch by providing a different plant and model.

- Changing optimiser settings: Modify the MPCConfig passed to the scenario; you can adjust tolerances, iteration limits, or even swap SLSQP for another method (requires modifying the minimize call).

- Adding a state observer: Replace the simple bias update with a Kalman filter or moving horizon estimator; the controller’s control method can be overridden.

- Multiple models for prediction: The model argument in MIMOMPC.__init__ determines the prediction model; the plant can be a different instance. This is already demonstrated in the CSTR mismatch scenario.

### 5.2 Practical Tuning Tips

- Horizons: Longer prediction horizon captures more future dynamics but increases problem size. Control horizon much smaller than prediction horizon reduces variables while still allowing aggressive moves early. Typical rule of thumb: Np ≥ (time constant + dead time) / dt, Nc ≈ Np/3.

- Weights: Start with output weights ~1 and move weights ~0.1. Increase move weights to dampen aggressive control. Terminal weight helps avoid myopic behaviour at the end of the horizon; often set 2–5× output weight.

- Scaling: output_scale should represent the typical operating range (e.g., difference between low and high setpoint). input_scale should reflect the allowed range. These make the cost function dimensionless and improve numerical conditioning.

- Bias filter: α close to 1 makes bias follow measurement quickly (good for accurate models); α small smoothes noise but may delay correction. Typical 0.1–0.3.

- Soft constraints: Use when hard output constraints might cause infeasibility due to model mismatch or aggressive setpoint changes. Set soft_output_weights high (e.g., 1e4) to strongly penalise violations, with maximum_output_slack as the absolute maximum allowed relaxation.

### 5.3 Reproducibility

The closed-loop simulation uses a fixed random seed. All randomness (measurement noise) is drawn from numpy.random.default_rng(seed). Therefore, runs are completely deterministic unless the code or dependencies change. This is invaluable for debugging and benchmarking.

### 5.4 Known Quirks

- Matplotlib may block script execution on some backends; use --no-plots for headless runs.

- If the CSTR integration blows up (temperature ≤ 1 K), the controller catches the exception and treats the candidate as invalid, often leading to fallback. This is intentional—the safety fallback prevents the real process from receiving extreme inputs.

- The program suppresses warnings during each SLSQP call to avoid cluttering output, but any warnings are captured and appended to the result message.

### 5.5 Where to Go Next

- Read the docstrings inside the source code; they are extensive.

- Modify the setpoint profile to see how MPC handles aggressive changes.

- Introduce your own plant model and test whether MPC can control it.

- Experiment with mismatched plant/model pairs and observe the bias correction at work.

- Attempt to improve performance by tuning weights or adding a terminal constraint.